How Do You Know If Your Game Has A Good Steering Wheel Response?

Note: this document applies to games that are more on the simulation side than the arcade side.

Ever wondered why some people can drive a real car without a problem but are unable to simply drive in a straight line using a steering wheel in a game?

The reason is very simple: it is because in many games the steering response is very different from that of a real car. The key for good steering wheel response in a game is to make it as similar as possible to that of a real car.

Most important facts about a real car's steering response:

- there is no dead zone

- the steering ratio is about 15:1 for sports cars to about 18:1 for family cars.

Note: A steering ratio of 15:1 means that turning the car's steering wheel by 15 degrees will have the effect of turning the car's front wheels by 1 degree.

Dead zone

DO NOT implement a dead zone. Dead zones were made for game pads specifically and they have no reason to be included in steering wheels. Dead zones will always make driving with a steering wheel more difficult.

Steering ratio

The next rule is to try to make the steering ratio as close as possible to that of a real car. Because a gaming wheel does not turn more than 200 degrees lock-to-lock (except the upcoming Logitech Driving Force Pro - which is 900 degrees!), it is not possible to directly map the steering wheel's position to the car's front wheel's angle. If you were to do that and we suppose that your car's front wheels turn 60 degrees lock-to-lock, then we would end up with a steering ratio of 200 / 60 = 3.33:1!!!! This is so far off the steering ratio of a real car that it will make it very difficult for most people to play the game! (That's a much quicker ratio than a Formula 1 car!)

Some games make the steering ratio even worse by only using half of the range of motion of the wheel... Always use the full range of the wheel!!

The way around the tight steering ratio problem is to make the sensitivity of the wheel dependant on speed (similar to what is usually done with a game pad). In order to achieve a steering ratio of 16:1 with a 200 degrees steering wheel, the car's front wheels need to turn 200 / 16 = 12.5 degrees lock to lock.

The optimal way to implement the steering response in your game is to check what the minimal speed is where 12.5 degrees lock to lock for your car's front wheels is acceptable. If for example such a speed is 30 mph, then you can program your game in a way that the car's front wheels lock-to-lock angle linearly scales from 60 degrees lock-to-lock at speed = 0 mph to 12.5 degrees lock-to-lock at speed = 30 mph and beyond. That way as long as you are driving above 30 mph your game will have the same steering response than that of a real car.